# Effects of Riparian Grazing on Distinct Phosphorus Sources

Alexander J Koiter

Tamaragh Y Malone

Abstract

Riparian areas play an important role in maintaining water quality in agricultural watersheds by buffering sediment, nutrients, and other pollutants. Recent studies have shown that riparian areas are less effective as buffers and, in some cases, are a net source of phosphorus (P) in cold climates. This study assessed the impact of cattle grazing or harvesting of riparian areas on the spatial and vertical distribution of P. This study measured the water-extractable phosphorus (WEP) in four distinctive sources: biomass, litter, organic layer, and Ah horizon in three riparian locations extending from the edge of the waterbody to the the field edge. Four treatments were examined: 1) control; 2) grazing; 3) high density grazing; and 4) mowing. Prior to implementing the treatments, the Ah (0-10cm) soil was the largest pool of WEP (42.5 mg m-2, ~44%); however, the biomass (i.e., standing vegetation) was a considerable proportion of the total (26.3 mg m-2, ~25%) WEP pool. The litter and organic layer had median P amounts of 11.1 and 17.7 mg m-2, respectively. Findings revealed significant reductions in biomass WEP with median reductions of 10.4 and 18.7 mg m-2 for high-density grazing and mowing treatments, respectively. This reduction was more pronounced in the lower riparian locations where there was more biomass available to be grazed or mowed. There were no detectable changes in the other sources of WEP across all the treatments. Assessment of the control plots (pre- and post-treatment) clearly indicate that there is considerable small-scale spatial variability in P measurements in riparian areas. Overall, the results of this study suggests that management practices that target vegetation, including harvesting and autumn short-term grazing, may be mechanisms to reduce the potential P loss during the snowmelt period. Studies investigating other important riparian processes that also have a demonstrated impact on the P mobility including freeze-thaw cycles and flooding are needed to fully assess the risk of P loss.

Core ideas

- Biomass and litter are substantial sources of WEP in riparian areas

- Autumn cattle grazing and mowing treatments reduced the amount of WEP in riparian biomass

- No Measurable change in the amount of WEP in the litter, organic layer, or Ah horizon post grazing

- Large spatial variability in WEP exists in riparian areas

Effects of Riparian Grazing on Distinct Phosphorus Sources

Alexander J Koiter*1 and Tamaragh Y Malone2

*corresponding author

1Brandon University, Department of Geography & Environment
270 18th St, Brandon, MB Canada, R7A 6A9
email: koitera@brandonu.ca
ORCID: 0000-0002-9355-9561

2Brandon University, Department of Biology, 270 18th St, Brandon, MB Canada, R7A 6A9
email: malonety35@brandonu.ca
ORCID: 0009-0002-5887-5574

Abbreviations

FTC, freeze-thaw cycle; MBFI, Manitoba Beef and Forage Initiatives; P, phosphorus; WEP, water extractable phosphorus

## Abstract

Riparian areas play an important role in maintaining water quality in agricultural watersheds by buffering sediment, nutrients, and other pollutants. Recent studies have shown that riparian areas are less effective as buffers and, in some cases, are a net source of phosphorus (P) in cold climates. This study assessed the impact of cattle grazing or harvesting of riparian areas on the spatial and vertical distribution of P. This study measured the water-extractable phosphorus (WEP) in four distinctive sources: biomass, litter, organic layer, and Ah horizon in three riparian locations extending from the edge of the waterbody to the the field edge. Four treatments were examined: 1) control; 2) grazing; 3) high density grazing; and 4) mowing. Prior to implementing the treatments, the Ah (0-10cm) soil was the largest pool of WEP (42.5 mg m-2, ~44%); however, the biomass (i.e., standing vegetation) was a considerable proportion of the total (26.3 mg m-2, ~25%) WEP pool. The litter and organic layer had median P amounts of 11.1 and 17.7 mg m-2, respectively. Findings revealed significant reductions in biomass WEP with median reductions of 10.4 and 18.7 mg m-2 for high-density grazing and mowing treatments, respectively. This reduction was more pronounced in the lower riparian locations where there was more biomass available to be grazed or mowed. There were no detectable changes in the other sources of WEP across all the treatments. Assessment of the control plots (pre- and post-treatment) clearly indicate that there is considerable small-scale spatial variability in P measurements in riparian areas. Overall, the results of this study suggests that management practices that target vegetation, including harvesting and autumn short-term grazing, may be mechanisms to reduce the potential P loss during the snowmelt period. Studies investigating other important riparian processes that also have a demonstrated impact on the P mobility including freeze-thaw cycles and flooding are needed to fully assess the risk of P loss.

## 1 Introduction

The increasing frequency and extent of algal blooms are typically linked to increased nutrient loading into lake and rivers. Phosphorus (P) loading is particularly concerning as this is generally the limiting nutrient in fresh water systems (Schindler et al., 2012). There have been many lab and field studies demonstrating the role and functionality of riparian areas in reducing P loading to surface water in agricultural settings (Yu et al., 2019). Infiltration, absorption, biological uptake, microbial activity, and sedimentation are the key processes that intercept and buffer the delivery of P (Lacas et al., 2005; Owens et al., 2007; McGuire and McDonnell, 2010). Convergence within the landscape coupled with climatic/weather conditions creates variability in hydrologic conditions and pathways, reducing the buffering capacity of riparian areas and ultimately resulting in reduced, inconsistent, and/or unsustainable reductions in P loading relative to many controlled experimental studies (Roberts et al., 2012; Habibiandehkordi et al., 2017).

In cold climates, the reduced infiltration due to frozen ground, limited vegetation uptake, and low microbial activity coupled with a flashy hydrograph during snowmelt creates conditions that further compromise the buffering capacity of riparian areas (Kieta et al., 2018; Nsenga Kumwimba et al., 2023). Additionally, research increasingly shows that riparian areas can contribute P (i.e., net source) to the surrounding environment (Roberts et al., 2012). The sources of this riparian-derived P are soil and vegetation. As soil P content increases, so does the risk of P loss through leaching and runoff (Habibiandehkordi et al., 2019). Soil P release can be intensified during periods of inundation that often occur dufing the spring snow melt, due to both to a longer period of soil-water contact and an increase solubility of iron-bound P as soil redox conditions lower (i.e., become anaerobic) (Carlyle and Hill, 2001; Young and Briggs, 2008). Vegetation P can become more mobile through the mineralization of P from decaying vegetation near the soil surface. There is also evidence that the longer vegetation-water contact during periods of inundation will also increase the amount of P leached out of the the dead vegetation contribute to the P available to be lost during runoff (Lozier and Macrae, 2017; Liu et al., 2019b). Both the soil and vegetation P sources can also be affected by freeze-thaw cycles (FTC). Repeated FTCs result in the cell disruption of microbial and plant biomass, releasing inter-cellular P to the surrounding environment (Kieta and Owens, 2019).

Management of riparian areas to maintain or enhance the buffering capacity of P is typically needed in the long term. Unlike nitrogen (N) where N can be significantly lost to the atmosphere through the processes of nitrification and denitrification to offset the continued input (Lyu et al., 2021), P is only generally lost through runoff or leaching. Harvesting and removing of biomass from the riparian area can be a practice to remove P and use the biomass for forage. However, mechanized biomass harvesting may be impractical or unsafe due to steep gradients, wet soil, and other obstacles like trees. Livestock grazing in riparian areas (riparian pastures) is common in the Canadian Prairies due to the abundance of forage, particularly during drought. Livestock exclusion from riparian areas has been suggested as a best management practices to reduce the direct inputs of P, limit bank erosion, and avoid soil compaction (Krall and Roni, 2023). However, stategies including alternative water sources, rotational grazing, timed-controlled grazing, rest-rotation grazing, and corridor fencing can all reduce those risks (Fitch et al., 2003).

From a surface water quality perspective, understanding the near-surface P distribution, both vertically and longitudinally, will help develop and identify best management practices for reducing P loading from riparian areas. Vertically, there are often four distinctive and identifiable sources of near-surface P: 1) biomass consisting of living standing vegetation; 2) litter consisting of fresh (~1-3 yrs) residues; 3) partially to well decomposed organic material; and 4) mineral soil (Reid et al., 2018). Longitudinally there often is a strong soil moisture gradient extending from the edge of the waterbody to the field edge. This results in changes biomass and litter including amount and composition as well as soil properties including organic matter content and horizon thickness. A better understanding of the spatial variability and relative contributions of the different sources of P is needed to assess the risks and benefits of different management strategies.

Given the timing and processes of P dynamics within riparian areas in cold climates, like the Canadian Prairies, reducing the near-surface concentration of soluble P prior to spring snowmelt would be a strategy to limit the contribution of P from the riparian area to surface water. Therefore, the overall aim of this study is to assess the impacts of short-term autumn cattle grazing and mowing on the sources and distribution of P in riparian areas. The objectives of this study were to quantify 1) the vertical profile of WEP using four distinctive P sources: biomass, litter, organic layer, and Ah horizon; 2) each of the four distinctive P sources in three riparian locations, near the edge of the waterbody (lower), close to the field edge (upper), and in between (middle); and 3) the net change in each of the four sources of WEP in each riparian location in response to grazing, high density grazing, and mowing (harvesting) of biomass. Understanding how riparian management practices affect the different sources of P can be used to help tailor management strategies in cold climates and ultimately reduce P loss and improve downstream water quality.

## 2 Methods

### 2.1 Site description

Source: Article Notebook

A randomized complete block experimental design was used to assess the sources of riparian P and investigate how it changes following cattle grazing or mowing treatments. The four treatments included a control, graze, high density graze, and mowing. Each treatment was replicated in riparian areas surrounding four separate Prairie potholes (wetlands). Samples of each unique source of P, biomass, litter, organic layer, and Ah horizon, were collected in three locations (upper, mid and lower) pre- and post-treatment. All samples were analyzed for WEP and the net change in each of the four distinctive sources of P was evaluated. The study was replicated across three sequential years using the same plots.

The study was conducted at the Manitoba Beef and Forage Initiatives (MBFI) research farm (50.06​∘N, 99.92​∘W; 502 AMSL), approximately 25 km north of Brandon, Manitoba, Canada, in the Prairie Pothole region of North America (Figure 1). The normal (1981 – 2010) average daily air temperature was 2.2 ​∘C, and the cumulative annual precipitation at Brandon was 474.2 mm, with 24.8 % falling as snow (Environment and Climate Change Canada, 2024). The Köppen-Geiger climate classification is cold, without dry season, and with warm summer (Dfb) (Beck et al., 2018). The region is predominately has agricultural land use, including annual crops (grains and oil seeds) and grazing/forage. MBFI is a 260 hectare (ha) research and demonstration farm with a mix of pasture, hay, and forage/silage cropland. Prior to the establishment of MBFI the site was part of the Manitoba Zero Tillage Research Association farm (1993-2014) where annual crops, including oil seeds and grains, were grown. There are also numerous small permanent and ephemeral wetlands (potholes) and associated riparian areas which account for ~35% of the total farm land (Manitoba Beef & Forage Initiatives, 2024). The riparian areas surrounding the larger permanent wetlands are fenced off to exclude livestock and are not actively managed. Approximately half the farm has an irregular undulating to hummocky relief (2-5%) with the reminder being nearly level (0-2%). The soils have developed on fine loamy, moderately calcareous glacial till. The drainage class in upper slope positions are well to rapidly draining while lower slope and riparian soils are poorly drained and primarily consist of Humic and Luvic Gleysols. The surface texture class of the riparian soil is a clay loam and pH values range from 7.1 to 8.3 with a mean of 7.6. Generally the surface soil profile can be described by a 1-10 cm organic layer overlying a 10-18 cm Ah horizon (Podolsky and Schindler, 1993). The vegetation in the riparian was assessed using the the foliar cover method for each plot within each of the four riparian areas. There was considerable variability between riparian areas, plots, and sampling locations (upper, mid and lower). The four most dominant species identified were Sow Thistle (Sonchus arvensis), Smooth Aster (Aster laevis), Kentucky bluegrass (Poa pratensis), and Smooth Brome (Bromus inermis) and the complete assessment can be found in Figure S1. All riparian areas investigated in this study were adjacent to actively grazed pastures.

Figure 1: Showing a) the location of the study site in southern Manitoba with an inset map of Canada; and b) the locations of the four riparian areas included in this study

Source: Map of study area

### 2.2 Experimental design

Four riparian areas surrounding permanent wetlands were selected (Figure 1) and were subdivided into four ~450 m2 plots. Within each riparian area, each plot was randomly assigned a treatment. The treatments consisted of 1) control, 2) graze, 3) high density graze, and 4) mow and harvest. The grazing treatments consisted of a five-hour grazing period, with the grazing treatment having ~3.1-3.5 animal units (AU) per plot and the high-density grazing with ~11.75-12 AU. For the mowing treatment, the vegetation was cut to a height of ~10cm, and the vegetation was manually raked out of the plot. The cattle were rotated daily over four consecutive days among the four riparian areas and the grazed plots were fenced on all four sides, including the edge of the waterbody, and provided with supplemental water. Treatments were applied early to mid September, before the first frost, in three consecutive years (2019-2021) (Figure S2) Within each plot three distinctive sampling locations, or landscape positions, were established, adjacent to the edge of the waterbody (Lower), adjacent to the field/pasture (Upper), and the mid-point (Mid). Samples were collected in each plot and sampling location 1-3 days prior and immediately adjacent 1-3 days following the treatments (including the control) to assess the impact of grazing and mowing.

### 2.3 Sampling and analysis

Four types of samples were collected: 1) biomass, 2) litter, 3) organic layer, and 4) Ah horizon. Using a 0.25 m2 quadrate, biomass was collected by cutting the standing live vegetation and litter by raking the surface and picking up the previous years growth. Both the biomass and litter were dried at 40 ​∘C, weighed, and homogenized using a blade grinder (<1cm). A composite of five soil samples was collected within the same quadrate as the biomass/litter using a 19 mm diameter soil probe and was divided into the organic layer and the top 10 cm of the Ah horizon. The organic layer and Ah soil were air-dried, disaggregated with a mortar and pestle, and passed through a 2-mm sieve. Additional bulk density samples of both the organic layer and Ah and the depth of the organic layer were collected in 2023. Daily air temperature and rainfall data were collected from an onsite station (Figure S2) (Manitoba Agriculture, 2023).

Water Extractable Phosphorus (WEP), an environmental soil and vegetation P test, was used to mimic soil P release to runoff water. Dried and homogenized samples were extracted by shaking (150 RPM) with deionized water for one hour at a mass to volume ratio of 1:30 for the biomass and litter samples (1 g) and 1:15 for the organic and Ah samples (2 g). Extractions were gravity filtered through a Whatman 42 filter followed by syringe filtration with a 0.45 μm nylon filter. WEP in the extract was measured spectrophotometrically by the colorimetric molybdate–ascorbic acid method (Murphy and Riley, 1962; Sharpley et al., 2006).

The concentration of WEP in the biomass and litter combined with the mass of material collected from the quadrate was used to calculate the total WEP (mg kg−1). Only the change in concentration (mg kg−1) was measured for the organic layer and Ah horizon. The vertical profile of WEP within the riparian area was assessed using samples collected before treatments were implemented across the 3-year study. The total WEP in the organic layer and Ah were estimated using the bulk density and depth measurements collected in 2023 (Figure 2 b).

### 2.4 Statistical analysis

All statistical analysis, plotting, and mapping was undertaken using the R Statistical Software (v4.4.0; R Core Team (2024)), through the RStudio Integrated Development Environment v2023.12.1.402 (RStudio, 2024). All plots and maps were created using the R package ggplot2 (v3.5.1; Wickham (2016)). Country and regional maps were created using data from the rnaturalearth package (Massicotte and South, 2023) and other maps using ESRI imagery and the OpenStreetMap package (Fellows, 2023). Generalized Linear Mixed Models (R package glmmTMB v1.1.9; Brooks et al. (2017)) were used to investigate the relation between the change in WEP (before - after treatment) and treatment and riparian sampling location for each of the four sources of WEP. Year and riparian area were included as crossed random factors to control for the variability between years and riparian areas. Additionally, when investigating the change in biomass WEP the WEP prior to the treatment was included as a covariate because the magnitude of the difference (i.e., before - after) is directly related to the amount initially available.

The interaction term was removed if there were no significant interactiona between the main effects (p <0.5). When a main effect or interaction were significant post-hoc pairwise comparisons with a Benjamini-Hochberg p-value adjustment was used (emmeans v1.10.1; Lenth (2024)). Model assumptions were assessed using DHARMa residual plots (DHARMa v0.4.6; Hartig (2022)), main effects were tested for collinearity (performance v0.12.2; Lüdecke et al. (2021)), and results were presented as type III ANOVA (car v3.1.2; Fox and Weisberg (2019)). For each unique source of WEP, the null hypotheses are that there is no difference in the net WEP between treatments or riparian sampling locations and there is no interaction between these two factors.

## 3 Results and Discussion

### 3.1 Vertical and longitudinal profiles of P

The four distinctive sources of P demonstrate is strong vertical stratification in both the concentration and total WEP (Figure 2). The median concentrations in the vegetation sources were 82.8 and 39.0 mg kg−1 for the biomass and litter components, respectively, which is more than an order of magnitude greater than the soil components (0.9 and 3.4 mg kg−1; Ah and organic, respectively). Considerable variability in the WEP concentration in the biomass and litter sources were observed with interquatile ranges (IQR) of 54.3 and 32.9 mg kg−1 for the biomass and litter sources, respectively. In contrast, the IQR for the organic and Ah sources was <2.5 mg kg−1. Overall, in terms of the total amount of WEP, the top 10 cm of the Ah horizon is the largest source of WEP (42.5 mg m−2) followed by the biomass (26.3 mg m−2), organic layer (14.3 mg m−2), and lastly the litter (13.7 mg m−2). The vertical profile of WEP in riparian areas (Figure 2) observed in this study supports the concept that a soil test P alone is likely missing a large proportion of the near-surface P that can be potentially lost during the spring snowmelt (Liu et al., 2019a; b; Cober et al., 2019). The substantial proportion of WEP above the soil surface provides evidence that managing the biomass in riparian areas in autumn may reduce the contribution of P lost directly from this area during spring. Specifically, the harvesting of this biomass results in an export of P which can maintain or enhance the buffering or storage capacity of P derived from upslope sources further improving downstream water quality (Kelly et al., 2007; Hille et al., 2019).

Figure 2: Vertical and longitudinal profiles of a) WEP concentration and b) WEP content in the riparian areas prior to grazing and mowing treatments.

Source: Vertical profile of WEP

The median concentrations are similar between the upper, mid, and lower positions in the biomass, organic, and Ah sources. An increase of ~20 mg kg−1 in WEP is observed in the litter from the upper to lower riparian sampling location. The total WEP does show an impact of the location within the riparian area. For the biomass and litter sources the lower riparian locations had greater amounts of WEP whereas the organic and Ah sources had greater amount of WEP in the upper riparian locations. The amount of variability is greatest in the Ah (IQR = 32.0 mg kg−1) and biomass (IQR = 23.3 mg kg−1) sources. The variability of the other two sources were similar with IQRS of 15.6 and 14.3 mg kg−1 for the litter and organic layer, respectively. The longitudinal gradient of WEP shows an inverted symmetry where the biomass WEP is largest near the lower sampling location and the Ah soil WEP is larger in the upper sampling location adjacent to the fields (Figure 2 b). The high soil water content in the lower location creates conditions that favor high biomass production (Figure S3) coupled with high biomass WEP concentrations (Figure 2 a) resulting in a considerable source of P. The higher amount of WEP in the Ah soil in the upper locations of the riparian area is due to the higher bulk density (Figure S3) and higher WEP concentration (Figure 2 a). The higher bulk density is most likely due to the lower soil organic matter content and the higher WEP concentration may be related to the interception of P-rich runoff from upslope areas (Tomer et al., 2007). Understanding and quantifying the sources and patterns of P within riparian areas is a key part of assessing the risk of P loss and designing effective management plans (Reid et al., 2018).

### 3.2 Impacts of grazing and mowing on P sources

Results of the ANOVA show a significant effect of treatment on the net biomass WEP (X2 = 24.8, df = 3, p < 0.001) and riparian location (X2 = 15.7, df = 2, p < 0.001). The net biomass WEP for the high-density grazing and mowing treatments were similar (p>0.05) but significantly (p<0.05) different from the control and graze treatments (Figure 3 a and Table 1). The mowing and high density grazing reduced the average WEP amount by 7.4 and 4.2 mg m−2 relative to the control, respectively. The reduction in biomass WEP was significantly (p<0.05) greater in the lower sampling locations as compared to the upper and mid locations (Figure 3 b and Table 1) with a difference in average WEP of 10.2 mg m−2 between the lower and upper locations of the riparian area.

Figure 3: Change in riparian biomass WEP following grazing or mowing in each riparian location. Within each plot significant differences (p<0.05) between treatments or riparian locations are denoted with different letters. Lower sampling locations are adjacent to the edge of the waterbody and Upper locations are adjacent to the field.

Source: Riparian vegetation WEP in response to grazing

Table 1: Results of the post-hoc pairwise comparisons with a Benjamini-Hochberg p value adjustment for differences in the net biomass WEP (mg m−2) between the four treatments and three riparian sampling locations. Contrast; Estimate; SE; df; t ratio; p value. Treatment. Control - High Graze; −4.83; 2.42; 132; −2.00; 0.072. Control - Mow; −8.52; 2.42; 132; −3.52; 0.002. Control - Regular Graze; 2.47; 2.40; 132; 1.03; 0.306. High Graze - Mow; −3.69; 2.43; 132; −1.51; 0.159. High Graze - Regular Graze; 7.30; 2.42; 132; 3.02; 0.006. Mow - Regular Graze; 10.99; 2.42; 132; 4.55; <0.001. Location. Lower - Middle; −7.94; 2.43; 132; −3.26; 0.002. Lower - Upper; −9.82; 2.57; 132; −3.83; <0.001. Middle - Upper; −1.87; 2.11; 132; −0.89; 0.377.

Source: Riparian vegetation WEP in response to grazing

There were no significant impacts of either treatment (X2 = 1.15, df = 3, p = 0.23) or riparian location (X2 = 4.30, df = 2, p = 0.56) on the amount of litter WEP (Figure 4). With respect to the WEP concentration in the organic layer the ANOVA detected no significant difference between riparian location (X2 = 0.57, df = 2, p = 0.75) but a significant (X2 = 8.24, df = 3, p = 0.04) effect of treatment. However, the post-hoc pairwise comparisons (Table 2) found no significant differences (p <0.05) between the three riparian positions. Lastly, there was no significant effect of treatment (X2 = 2.59, df = 3, p = 0.46) or riparian position (X2 = 1.17, df = 2, p = 0.56) in the concentration of WEP in the top 10 cm of the Ah horizon (Figure 6). There was considerable variation across all treatments and riparian locations in all four P sources. This high variability in WEP amount/concentration is best reflected in the control treatment where the expected difference is 0. Still, WEP losses and gains were measured despite no treatment being applied.

Figure 4: Change in riparian litter WEP following grazing or mowing in each of the riparian locations. No significant effect of treatment or riparian location on the litter WEP content was detected. Lower sampling locations are adjacent to the edge of the waterbody and Upper locations are adjacent to the field.

Source: Riparian litter WEP in response to grazing

Figure 5: Change in riparian organic layer WEP concentration following grazing or mowing in each of the riparian locations. A significant effect of treatment was detected; however, the post-hoc analysis was not able to detect any significant (p < 0.05) pairwise contrasts. Lower sampling locations are adjacent to the edge of the waterbody and Upper locations are adjacent to the field.

Source: Riparian organic and mineral soil WEP in response to grazing

Table 2: Results of the post-hoc pairwise comparisons with a Benjamini-Hochberg p value adjustment for differences in the net organic layer WEP (mg kg−1) between the four treatments. Contrast; Estimate; SE; df; t ratio; p value. Control - Graze; −1.49; 0.59; 135; −2.50; 0.066. Control - High Graze; −0.63; 0.59; 135; −1.05; 0.353. Control - Mow; −1.38; 0.59; 135; −2.32; 0.066. Graze - High Graze; 0.86; 0.59; 135; 1.45; 0.299. Graze - Mow; 0.11; 0.59; 135; 0.18; 0.856. High Graze - Mow; −0.75; 0.59; 135; −1.27; 0.311.

Source: Riparian organic and mineral soil WEP in response to grazing

Figure 6: Change in riparian Ah layer (0-10cm) WEP concentration following grazing or mowing in each of the riparian locations. No significant effect of treatment or location was detected. Lower sampling locations are adjacent to the edge of the waterbody and Upper locations are adjacent to the field.

Source: Riparian organic and mineral soil WEP in response to grazing

The results of this study suggest that short-term autumn high density grazing may be a potential management tool that can reduce the amount of P lost directly from the riparian area (Figure 3 a). In addition to managing P loss, grazing riparian areas can also provide an essential source of forage, particularly during drought. Mechanized harvesting of biomass will also achieve this reduction in P loss (Figure 3 a) if the landscape and soil conditions are favorable. Despite the cycling of nutrients by the removal of P through grazing of biomass (Figure 3) and the deposition through excretion no differences were detected in the litter and Ah sources of P (Figure 4, and 6). The ANOVA did detect a significant effect of treatment on the organic layer WEP; however, the pairwise comparisons were not able to detect any significant differences and the exact nature of the impact of the treatments remains unclear. The ability to detect changes in the WEP sources in riparian areas is difficult due to spatial variability in both the inherent and post-grazing treatment. Even within the control plots, both net addition and removal of WEP were measured and in many cases the amount of variability was similar across treatments. This inherent variability (i.e., pre-grazing) is a result of a combination of hydrological factors like ground water fluctuations, soil attributes such as texture, ecological dynamics involving plant community composition, and anthropogenic influences like historical land management practices (McClain et al., 2003; Vidon et al., 2010). In particular, the species cover information (Figure S1) demonstrates a wide range in species composition and abundance, this coupled with the variation in P release with different vegetation species may explain some of the observed variability (Cober et al., 2018).

### 3.3 Sources of variability and uncertainty in P sources

The Prairie pothole wetlands regularly experience high water levels in the early spring. Observations made adjacent to one plot between Oct 2020 and May 2021 showed that the lower, mid, and upper sampling points would have experienced inundation for approximately 21, 11, and zero days, respectively. The annual weather conditions and topography of riparian areas surrounding the wetlands will have an large impact on the length and extent of flooding. Prolonged contact with water has been shown to increase the amount of WEP in both soil (Young and Briggs, 2008) and vegetation (Lozier and Macrae, 2017) and also may explain some of the observed variability. As reported by Podolsky and Schindler (1993), the soils surrounding these potholes are typically low in CaCo3 and have a neutral to slight alkaline pH. In this pH range (~6.5 to 7.5) P availability is typically at its highest and not expected to precipitate with Ca. A more detailed soil chemical analysis, particularly Fe and Mn, along with soil saturation duration information (i.e., redox) is needed to fully assess the potential for P loss during the spring (Walton et al., 2020). The WEP protocol used for both soil and vegetation samples are not likely to capture mobilize redox-sensitive P from the soil (Walton et al., 2020) or enhanced P leaching from vegetation (Lozier and Macrae, 2017). Similarly, the WEP protocol also does not capture the enhanced P release from soil and vegetation that results repeated freeze-thaw cycles (Liu et al., 2013; Lozier and Macrae, 2017). Temperature sensors placed at the soil surface adjacent to one plot recorded four freeze-thaw cycles between Oct 2020 and May 2021, surface temperatures fluctuations are moderated in this region by the continental climate and relatively persistent snow pack. Both the prolonged contact with water and freeze-thaw cycles are are not captured in the WEP protocols and likely result in an underestimation of the potential for P loss from the each of the four distinctive source of P in riparian areas.

Post-grazing treatment, the added urine and manure create additional hotspots of P that may carry forward to subsequent years (Subedi et al., 2020; Donohoe et al., 2021). The single 0.25 m2 sampling quadrate within each riparian location may have been insufficient to capture the spatial variability. Therefore, larger composite and/or several sampling locations within each upper, middle and lower locations are recommended. Appropriate sampling design becomes critical as the scale of observation of similar research increases to the farm scale, and so will the amount and source of variability. As the scope of research is expanded to the farm level, the importance of using an appropriate sampling design becomes increasingly critical (Hale et al., 2014).

### 3.4 Managment implications

Autumn was selected for the mowing and grazing treatments for three reasons. The first was to reduce the amount of biomass P available that can contribute to the P loss during the spring snowmelt. Secondly, the drier soil conditions reduce the amount of pugging and soil compaction, which limits the disruption of soil structure and damage to plants (Batey, 2009). Lastly, the prairie potholes and associated riparian areas are important breeding habitats for migratory birds, and late-season grazing may reduce the ecological impact (Stanley and Knopf, 2002). However, the type of grazing system (timing, stocking rate, and density, etc.) may impact habitat quality and breeding success (Carnochan et al., 2018; Hansen et al., 2019; Kraft et al., 2021). Corridor fencing at the edge of the waterbody and alternative water sources were used in this study to limit livestock access to prevent bank erosion and protect water quality (e.g., direct deposition) (Dauwalter et al., 2018). Scaling this up to the farm level might be expensive (fencing infrastructure) (Aarons et al., 2013) and time-consuming (short-term grazing), especially in prairie pothole regions where there are numerous and small riparian areas (Manitoba Agriculture, 2024). The long-term impacts of repeated grazing of riparian areas also needs to be considered. From a nutrient loss reduction perspective, a shift in the magnitude of P sources could be expected as less biomass is available to be added to the litter source, affecting the organic layer and Ah sources of P. The regular inclusion of cattle will also introduce a new manure source of P, which can spatially redistribute P and initially be more water soluble and readily transported (Franzluebbers et al., 2019). Grazing can also reduce the litter layer through trampling increasing the soil-vegetation contact, and speeding up the decomposition process. These changes in biomass and litter quantities may result in changes to habitat structure. Understanding how forage management directly impacts both the plant and soil P dynamics is important for understanding both the agronomic and environmental P considerations (Subedi et al., 2020).

## 4 Conclusion

Biomass and litter are significant sources of near-surface WEP in riparian areas that historically have been disregarded as necessary. Management of the biomass prior to the onset of winter conditions in cold climates has the potential to reduce the amount of P directly lost during the spring snowmelt and maintain or enhance the nutrient buffering capacity. The results from this experiment demonstrated that short-term high-density cattle grazing and mowing both resulted in a reduction in the amount of biomass WEP, particularly in the lower riparian locations. The grazing and mowing treatments had no detectable effect on the other three near-surface sources of WEP. However, detecting changes in the near-surface sources of WEP is challenging due to high spatial variability.

Comparatively less riparian research has occurred in landscapes that experience a cold climate with strong temperature seasonality (e.g., Canadian prairies). In these regions, the runoff and nutrient losses occur predominately during the spring snowmelt period. The repeated FTC of the vegetation and soils increases the potential P losses during this key time. Continued research to identify, quantify, and manage these sources of P to improve water quality remains a priority. In addition improving water quality, the development of riparian management strategies should prioritize the protection other ecological goods and services and recognize these areas as an integral part of the farm.

## Acknowledgements

This project was undertaken with the financial support of the Government of Canada through the federal Department of Environment and Climate Change and a Lake Winnipeg Basin Program grant awarded to the Manitoba Association of Watersheds. Additional research funding was provided through a Brandon University Research Committee grant awarded to AK. Thank you to A. Avila, M. Luna, C Sobchuk, and A. Tan for all the help with lab and field work. Special thanks to the Manitoba Beef and Forage Initiatives research farm staff for the use of their facilities and managing the cattle grazing and mowing treatments. Lastly, thank you to R. Canart and M. Elsinger for helping to develop the experimental design.

## Data availability

Data and source code for analysis and manuscript available on GitHub: https://github.com/alex-koiter/riparian-grazing-manuscript

## Conflict of interest statement

The authors have no competing interests to declare that are relevant to the content of this article.

## Author contributions

The authors confirm contribution to the paper as follows: study conception and design: A. Koiter; data collection: T. Malone; analysis and interpretation of results: A. Koiter; draft manuscript preparation: A. Koiter and T. Malone. All authors reviewed the results and approved the final version of the manuscript.

## References

Aarons, S.R., A.R. Melland, and L. Dorling. 2013. Dairy farm impacts of fencing riparian land: Pasture production and farm productivity. Journal of Environmental Management 130: 255–266. doi: 10.1016/j.jenvman.2013.08.060.

Batey, Tom. 2009. Soil compaction and soil management  a review. Soil Use and Management 25(4): 335–345. doi: 10.1111/j.1475-2743.2009.00236.x.

Beck, H.E., N.E. Zimmermann, T.R. McVicar, N. Vergopolan, A. Berg, et al. 2018. Present and future Köppen-Geiger climate classification maps at 1-km resolution. Scientific Data 5(1): 180214. doi: 10.1038/sdata.2018.214.

Brooks, M.E., K. Kristensen, K.J. van Benthem, A. Magnusson, C.W. Berg, et al. 2017. glmmTMB balances speed and flexibility among packages for zero-inflated generalized linear mixed modeling. The R Journal 9(2): 378–400. doi: 10.32614/RJ-2017-066.

Carlyle, G.C., and A.R. Hill. 2001. Groundwater phosphate dynamics in a river riparian zone: Effects of hydrologic flowpaths, lithology and redox chemistry. Journal of Hydrology 247(3): 151–168. doi: 10.1016/S0022-1694(01)00375-4.

Carnochan, S.J., C.C. De Ruyck, and N. Koper. 2018. Effects of twice-over rotational grazing on songbird nesting success in years with and without flooding. Rangeland Ecology & Management 71(6): 776–782. doi: 10.1016/j.rama.2018.04.013.

Cober, J.R., M.L. Macrae, and L.L. Van Eerd. 2018. Nutrient release from living and terminated cover crops under variable freezethaw cycles. Agronomy Journal 110(3): 1036–1045. doi: 10.2134/agronj2017.08.0449.

Cober, J.R., M.L. Macrae, and L.L. Van Eerd. 2019. Winter phosphorus release from cover crops and linkages with runoff chemistry. Journal of Environmental Quality 48(4): 907–914. doi: 10.2134/jeq2018.08.0307.

Dauwalter, D.C., K.A. Fesenmyer, S.W. Miller, and T. Porter. 2018. Response of riparian vegetation, instream habitat, and aquatic biota to riparian grazing exclosures. North American Journal of Fisheries Management 38(5): 1187–1200. doi: 10.1002/nafm.10224.

Donohoe, G., D. Flaten, F. Omonijo, and K. Ominski. 2021. Short-term impacts of winter bale grazing beef cows on forage production and soil nutrient status in the eastern canadian prairies. Canadian Journal of Soil Science 101(4): 717–733. doi: 10.1139/cjss-2021-0028.

Environment, and Climate Change Canada. 2024. Canadian Climate Normals. https://climate.weather.gc.ca/climate_normals/index_e.html.

Fellows, I. 2023. OpenStreetMap: Access to open street map raster images.

Fitch, L., B. Adams, and K. O’Shaughnessy. 2003. Caring for the green zone: Riparian areas and grazing management. 3rd ed. Cows; Fish Program, Lethbridge, Alberta.

Fox, J., and S. Weisberg. 2019. An R companion to applied regression. Third. Sage, Thousand Oaks CA.

Franzluebbers, A.J., M.H. Poore, S.R. Freeman, and J.R. Rogers. 2019. Soil-surface nutrient distributions in grazed pastures of North Carolina. Journal of Soil and Water Conservation 74(6): 571–583. doi: 10.2489/jswc.74.6.571.

Habibiandehkordi, R., D.A. Lobb, P.N. Owens, and D.N. Flaten. 2019. Effectiveness of vegetated buffer strips in controlling legacy phosphorus exports from agricultural land. Journal of Environmental Quality 48(2): 314–321. doi: 10.2134/jeq2018.04.0129.

Habibiandehkordi, R., D.A. Lobb, S.C. Sheppard, D.N. Flaten, and P.N. Owens. 2017. Uncertainties in vegetated buffer strip function in controlling phosphorus export from agricultural land in the canadian prairies. Environmental Science and Pollution Research 24(22): 18372–18382. doi: 10.1007/s11356-017-9406-6.

Hale, R., P. Reich, T. Daniel, P.S. Lake, and T.R. Cavagnaro. 2014. Scales that matter: Guiding effective monitoring of soil properties in restored riparian zones. Geoderma 228-229: 173–181. doi: 10.1016/j.geoderma.2013.09.019.

Hansen, B.D., H.S. Fraser, and C.S. Jones. 2019. Livestock grazing effects on riparian bird breeding behaviour in agricultural landscapes. Agriculture, Ecosystems & Environment 270-271: 93–102. doi: 10.1016/j.agee.2018.10.016.

Hartig, F. 2022. DHARMa: Residual diagnostics for hierarchical (multi-level / mixed) regression models.

Hille, S., D. Graeber, B. Kronvang, G.H. Rubæk, N. Onnen, et al. 2019. Management options to reduce phosphorus leaching from vegetated buffer strips. Journal of Environmental Quality 48(2): 322–329. doi: 10.2134/jeq2018.01.0042.

Kelly, J.M., J.L. Kovar, R. Sokolowsky, and T.B. Moorman. 2007. Phosphorus uptake during four years by different vegetative cover types in a riparian buffer. Nutrient Cycling in Agroecosystems 78(3): 239–251. doi: 10.1007/s10705-007-9088-4.

Kieta, K.A., and P.N. Owens. 2019. Phosphorus release from shoots of phleum pretense l. After repeated freeze-thaw cycles and harvests. Ecological Engineering 127: 204–211. doi: 10.1016/j.ecoleng.2018.11.024.

Kieta, K.A., P.N. Owens, D.A. Lobb, J.A. Vanrobaeys, and D.N. Flaten. 2018. Phosphorus dynamics in vegetated buffer strips in cold climates: A review. Environmental Reviews 26(3): 255–272. doi: 10.1139/er-2017-0077.

Kraft, J.D., D.A. Haukos, M.R. Bain, M.B. Rice, S. Robinson, et al. 2021. Using grazing to manage herbaceous structure for a heterogeneity-dependent bird. The Journal of Wildlife Management 85(2): 354–368. doi: 10.1002/jwmg.21984.

Krall, M., and P. Roni. 2023. Effects of livestock exclusion on stream habitat and aquatic biota: A review and recommendations for implementation and monitoring. North American Journal of Fisheries Management 43(2): 476–504. doi: 10.1002/nafm.10863.

Lacas, J.-G., M. Voltz, V. Gouy, N. Carluer, and J.-J. Gril. 2005. Using grassed strips to limit pesticide transfer to surface water: A review. Agronomy for Sustainable Development 25(2): 253–266. doi: 10.1051/agro:2005001.

Lenth, R.V. 2024. Emmeans: Estimated marginal means, aka least-squares means.

Liu, J., H.M. Baulch, M.L. Macrae, H.F. Wilson, J.A. Elliott, et al. 2019a. Agricultural water quality in cold climates: processes, drivers, management options, and research needs. Journal of Environmental Quality 48(4): 792–802. doi: 10.2134/jeq2019.05.0220.

Liu, J., R. Khalaf, B. Ulén, and G. Bergkvist. 2013. Potential phosphorus release from catch crop shoots and roots after freezing-thawing. Plant and Soil 371(1): 543–557. doi: 10.1007/s11104-013-1716-y.

Liu, J., M.L. Macrae, J.A. Elliott, H.M. Baulch, H.F. Wilson, et al. 2019b. Impacts of cover crops and crop residues on phosphorus losses in cold climates: a review. Journal of Environmental Quality 48(4): 850–868. doi: 10.2134/jeq2019.03.0119.

Lozier, T.M., and M.L. Macrae. 2017. Potential phosphorus mobilization from above-soil winter vegetation assessed from laboratory water extractions following freezethaw cycles. Canadian Water Resources Journal 42(3): 276–288. doi: 10.1080/07011784.2017.1331140.

Lüdecke, D., M.S. Ben-Shachar, I. Patil, P. Waggoner, and D. Makowski. 2021. performance: An R package for assessment, comparison and testing of statistical models. Journal of Open Source Software 6(60): 3139. doi: 10.21105/joss.03139.

Lyu, C., X. Li, P. Yuan, Y. Song, H. Gao, et al. 2021. Nitrogen retention effect of riparian zones in agricultural areas: A meta-analysis. Journal of Cleaner Production 315: 128143. doi: 10.1016/j.jclepro.2021.128143.

Manitoba Agriculture. 2023. Manitoba agriculture weather program. https://www.gov.mb.ca/agriculture/weather/manitoba-ag-weather.html.

Manitoba Agriculture. 2024. Agriculture rotational grazing. https://www.gov.mb.ca/agriculture/.

Manitoba Beef & Forage Initiatives. 2024. MBFI farm stations. https://www.mbfi.ca/farm-station.

Massicotte, P., and A. South. 2023. Rnaturalearth: World map data from natural earth.

McClain, M.E., E.W. Boyer, C.L. Dent, S.E. Gergel, N.B. Grimm, et al. 2003. Biogeochemical hot spots and hot moments at the interface of terrestrial and aquatic ecosystems. Ecosystems 6(4): 301312. doi: 10.1007/s10021-003-0161-9.

McGuire, K.J., and J.J. McDonnell. 2010. Hydrological connectivity of hillslopes and streams: Characteristic time scales and nonlinearities. Water Resources Research 46(10): W10543. doi: 10.1029/2010WR009341.

Murphy, J., and J.P. Riley. 1962. A modified single solution method for the determination of phosphate in natural waters. Analytica Chimica Acta 27: 31–36. doi: 10.1016/S0003-2670(00)88444-5.

Nsenga Kumwimba, M., J. Huang, M. Dzakpasu, K. De Silva, O.E. Ohore, et al. 2023. An updated review of the efficacy of buffer zones in warm/temperate and cold climates: Insights into processes and drivers of nutrient retention. Journal of Environmental Management 336: 117646. doi: 10.1016/j.jenvman.2023.117646.

Owens, P.N., J.H. Duzant, L.K. Deeks, G.A. Wood, R.P.C. Morgan, et al. 2007. Evaluation of contrasting buffer features within an agricultural landscape for reducing sediment and sediment-associated phosphorus delivery to surface waters. Soil Use and Management 23(s1): 165–175. doi: 10.1111/j.1475-2743.2007.00121.x.

Podolsky, G.P., and D. Schindler. 1993. Soils of the manitoba zero tillage research association farm. Winnipeg, Manitoba.

R Core Team. 2024. R: A language and environment for statistical computing. http://www.R-project.org.

Reid, K., K. Schneider, and B. McConkey. 2018. Components of phosphorus loss from agricultural landscapes, and how to incorporate them into risk assessment tools. Frontiers in Earth Science 6. https://www.frontiersin.org/article/10.3389/feart.2018.00135.

Roberts, W.M., M.I. Stutter, and P.M. Haygarth. 2012. Phosphorus retention and remobilization in vegetated buffer strips: A review. Journal of Environmental Quality 41(2): 389–399. doi: 10.2134/jeq2010.0543.

RStudio. 2024. RStudio: Integrated development environment for r. http://www.rstudio.org/.

Schindler, D.W., R.E. Hecky, and G.K. McCullough. 2012. The rapid eutrophication of lake winnipeg: Greening under global change. Journal of Great Lakes Research 38, Supplement 3: 6–13. doi: 10.1016/j.jglr.2012.04.003.

Sharpley, A.N., P.J.A. Kleinman, and J.L. Weld. 2006. Environmental soil phosphorus indices. In: Carter, M.R. and Gregorich, E.G., editors. 2nd ed. CRC Press, Boca Raton, FL, U.S.A

Stanley, T.R., and F.L. Knopf. 2002. Avian responses to late-season grazing in a shrub-willow floodplain. Conservation Biology 16(1): 225–231. doi: 10.1046/j.1523-1739.2002.00269.x.

Subedi, A., D. Franklin, M. Cabrera, A. McPherson, and S. Dahal. 2020. Grazing systems to retain and redistribute soil phosphorus and to reduce phosphorus losses in runoff. Soil Systems 4(4): 66. doi: 10.3390/soilsystems4040066.

Tomer, M.D., T.B. Moorman, J.L. Kovar, D.E. James, and M.R. Burkart. 2007. Spatial patterns of sediment and phosphorus in a riparian buffer in western iowa. Journal of Soil and Water Conservation 62(5): 329–338.

Vidon, P., C. Allan, D. Burns, T.P. Duval, N. Gurwick, et al. 2010. Hot spots and hot moments in riparian zones: Potential for improved water quality management. Journal of the American Water Resources Association 46(2): 278–298. doi: 10.1111/j.1752-1688.2010.00420.x.

Walton, C.R., D. Zak, J. Audet, R.J. Petersen, J. Lange, et al. 2020. Wetland buffer zones for nitrogen and phosphorus retention: Impacts of soil type, hydrology and vegetation. Science of The Total Environment 727: 138709. doi: 10.1016/j.scitotenv.2020.138709.

Wickham, H. 2016. ggplot2: Elegant graphics for data analysis. Springer-Verlag, New York NY U.S.A.

Young, E.O., and R.D. Briggs. 2008. Phosphorus concentrations in soil and subsurface water: A field study among cropland and riparian buffers. Journal of Environmental Quality 37(1): 69–78. doi: 10.2134/jeq2006.0422.

Yu, C., P. Duan, Z. Yu, and B. Gao. 2019. Experimental and model investigations of vegetative filter strips for contaminant removal: A review. Ecological Engineering 126: 25–36. doi: 10.1016/j.ecoleng.2018.10.020.

## Supplemental materials

Figure S1: Initial year (2019) cover assessment using the foliar cover method for each plot within the four riparian locations

Figure S2: Average daily air temperature and cumulative rainfall over the growing season over the three year study. Red bars indicate sampling dates

Figure S3: a) Mass of biomass and litter before grazing and mowing (2019-2021) and b) the bulk density of the organic layer and 10 cm Ah horizon (2023)